Technical Guide

Maintenance Schedule

February 16th 2026

| Reference |  | GuideTechnique-PTCTLT-EN |  |  |  | Rating |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accessibility |  |  | Accès réservé |  | Restreint |  |  | Interne |  |  | Libre (à préciser) | X |
| Summary |  |  |  |  |  |  |  |  |  |  |  |  |
| This document describes the format for exchanging data relating to the Short and Long term Maintenance Schedule (PTCT and PTLT) |  |  |  |  |  |  |  |  |  |  |  |  |

# Version tracking

| Version | Date | Autor(s) | Description |
| --- | --- | --- | --- |
| V1.0 | 20/05/2022 | M LE GAOUYAT | Initial Version |
| V1.1 | 26/09/2022 | M LE GAOUYAT | Modifications (API §3 & §6 ; §5.2 & PTLT File) |
| V1.2 | 10/10/2022 | C FLORESTANO | To clarify API Acess |
| V1.3 | 01/12/2023 | C FLORESTANO | Add mapping API |
| V1.4 | 16/02/2026 | C. FLAMANT | Modification of the header of PT files in force on 01/07/2026 |

# Description

The Maintenance Schedule is a common document, which shows the restriction data on the upstream network of NaTran. It is divided into 2 documents : short term (from D to D+1) and long term (from D+2 to D+n).

# Available method of publication

PT CT & LT documents are published by the following method :

- Proactive :

  - PT CT : a minima one publication per day before 14h for D and D+1 Gas Day in csv format. The file will be published by sFTP and downloadable from Ingrid.

  - PT LT : one publication for all the modifications from D+2 until the end. It’s real time update. The file will be published by sFTP and downloadable from Ingrid. The document concerns all the pcr/direction/GD of the perimeter (not only the modification).

- By API : An API is available to get the data related to the Maintenance Schedule Program (LT & CT) publication in JSON format. Credentials are required for the connection : please refer to your NaTran commercial contact.

# Document name and format

The file will be published on csv format, with:

- A semicolon as a list separator

- A comma as a decimal mark

The name of the file will comply with the following rules:

| N° | Label | Type | Length | Format |
| --- | --- | --- | --- | --- |
| 1 | Type of the document | Alphanumeric | 4 | PTCT, PTLT |
| 2 | Generation Date | Date | 17 | DDMMYYYYhhmmssSSS |
| 3 | Extension | Alphanumeric | 4 | .csv |
| 4 | Separators |  | 2 | « _ » |

As a result, the document will have for instance the following name:

PTCT_EXPEDITEUR_JJMMAAAAhhmmssSSS.csv

PTLT_EXPEDITEUR_JJMMAAAAhhmmssSSS.csv

# Description of document for each format

The file will be published on csv format.

# Maintenance Schedule Short Terme (PTCT)

Header :

The header contains the following data:

- Programme Travaux / Maintenance Schedule :

  - Example : PTC_CT-XXXXX

  - The reference is built with the following rules :

    - 5 letters PTC_CT

    - A sequence

- Réseau / Network :

  - Example : NaTran

- Période / Period :

  - Example : 01/12/2022 06 : 00 – 03/12/2022 06 :00

  - The period is calculated with the gas day inside the PT CT (min and max)

- Date de mise à jour / Last update

  - Example : 01/12/2022 13 :45 : 25

  - Update of PT CT

Data table

The tables presented in this part contain the following columns:

- Col No: number of the column in the row

- Name: description of the content of the field

- Type: field type

- Format: data format

- Mandatory: determines whether the field is mandatory or not;

- Description: additional precision

- Example: range of values that the data can take or examples of values.

- Mapping API : to define mapping csv data with api data

| N° Col | Nom | Type | Format | Oblig. | Description | Exemple | Mapping API consolidated-maintenance-programs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PCR / Service Point | Texte |  | O | Code of Contractual point | IR0006, IR0010, etc. | contractualPointCode |
| 2 | Libellé / Label | Texte |  | O | Name of Contractual point | Virtualys, Oltingue, … | contractualPointLabel |
| 3 | Sens / Direction | Texte |  | O | Direction of Contractual point | Rec, Del | direction |
| 4 | Journée gazière / Gas Day | Date | jj/mm/aaaa | O | Gas Day | 01/01/2022 | gasDay |
| 5 | TRf | Numérique |  | N | Firm restriction rate | 0,1234 | firmRestrictionRate |
| 6 | TRi | Numérique |  | N | Interruptible restriction rate | 0,1234 | interruptibleRestrictionRate |
| 7 | TRi A | Numérique |  | N | Annual Interruptible restriction rate | 0,1234 | annualInterruptibleRestrictionRate |
| 8 | TRi T | Numérique |  | N | Quarterly Interruptible restriction rate | 0,1234 | quarterlyInterruptibleRestrictionRate |
| 9 | TRi M | Numérique |  | N | Monthly Interruptible restriction rate | 0,1234 | monthlyInterruptibleRestrictionRate |
| 10 | TRi Q | Numérique |  | N | Daily Interruptible restriction rate | 0,1234 | dailyInterruptibleRestrictionRate |
| 11 | CTE | Numérique |  | O | Effective Technical Capacity | 100,000000 (en GWh/J 25°C) | availableTechnicalCapacity |
| 12 | CTM | Numérique |  | N | Mutual Technical Capacity (PITS only) | 100,000000 (en GWh/J 25°C) | minimalTechnicalCapacity |
| 13 | ∑COSf | Numérique |  | N | Sum of firm subscribed operational capacities | 100,000000 (en GWh/J 25°C) | firmAggregatedCos |
| 14 | ∑COSi | Numérique |  | N | Sum of interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | interruptibleAggregatedCos |
| 15 | ∑COSi A | Numérique |  | N | Sum of annual interruptible subscrided operational capacities | 100,000000 (en GWh/J 25°C) | annualInterruptibleAggregatedCos |
| 16 | ∑COSi T | Numérique |  | N | Sum of quaterly interuptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | quarterlyInterruptibleAggregatedCos |
| 17 | ∑COSi M | Numérique |  | N | Sum of monthly interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | monthlyInterruptibleAggregatedCos |
| 18 | ∑COSi Q | Numérique |  | N | Sum of daily interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | dailyInterruptibleAggregatedCos |
| 19 | CTNf | Numérique |  | N | Firm nominal technical capacity | 100,000000 (en GWh/J 25°C) | firmNominalTechnicalCapacity |
| 20 | Travaux Petits Impacts / Low Impact Maintenance | Booléen |  | O | Low Impact Maintenance |  | lowImpactMaintenance |
| 21 | Date et Heure de Mise à jour / Update date and time | Date/Heure | aaaa-mm-jjThh:mm:ssZ | O |  |  | updateDateTime |

File example :

# Maintenance Schedule Long Terme (PTLT)

Header :

The header contains the following data:

- Programme Travaux / Maintenance Schedule :

  - Example : PTC_LT-XXXXX

  - The reference is built with the following rules :

    - 5 letters PTC_LT

    - A sequence

- Réseau / Network :

  - Example : NaTran

- Période / Period :

  - Example : 01/12/2022 06 : 00 – 01/03/2023 06 :00

  - The period is calculated with the gas day inside the PT LT (min and max)

- Date de mise à jour / Last update

  - Example : 25/11/2022 09 :06 : 25

  - Update of PT LT

Data table

The tables presented in this part contain the following columns:

- Col No: number of the column in the row

- Name: description of the content of the field

- Type: field type

- Format: data format

- Mandatory: determines whether the field is mandatory or not

- Description: additional precision

- Example: range of values that the data can take or examples of values.

- Mapping API : to define mapping csv data with api data

| N° Col | Nom | Type | Format | Oblig. | Description | Exemple | Mapping API consolidated-maintenance-programs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PCR / Service Point | Texte |  | O | Code of Contractual point | IR0006, IR0010, etc. | contractualPointCode |
| 2 | Libellé / Label | Texte |  | O | Name of Contractual point | Virtualys, Oltingue, … | contractualPointLabel |
| 3 | Sens / Direction | Texte |  | O | Direction of Contractual point | Rec, Del | direction |
| 4 | Journée gazière / Gas Day | Date | jj/mm/aaaa | O | Gas Day | 01/01/2022 | gasDay |
| 5 | TRf Max | Numérique |  | N | Maximum restriction rate on the subscrided capacity | 0,1234 | maxFirmRestrictionRate |
| 6 | TRf Probable | Numérique |  | N | Probable restriction rate on the subscrided capacity | 0,1234 | probableFirmRestrictionRate |
| 7 | TRf Max Nominal | Numérique |  | N | Maximum restriction rate on the marketable capacity | 0,1234 | maxNominalFirmRestrictionRate |
| 8 | TRf Probable Nominal | Numérique |  | N | Probable restriction rate on the marketable capacity | 0,1234 | probableNominalFirmRestrictionRate |
| 9 | CPRTi | Numérique |  | N | Probable capacity during maintenance taking into account the interruptible | 100,000000 (en GWh/J 25°C) | interruptibleProbableTechnicalCapacity |
| 10 | CMNTt | Numérique |  | N | Minimum available capacity during maintenance | 100,000000 (en GWh/J 25°C) | totalMinimalTechnicalCapacity |
| 11 | CPRTt | Numérique |  | N | Probable capacity during maintenance | 100,000000 (en GWh/J 25°C) | totalProbableTechnicalCapacity |
| 12 | ∑COSf | Numérique |  | N | Sum of firm subscribed operational capacities | 100,000000 (en GWh/J 25°C) | firmAggregatedCos |
| 13 | ∑COSi | Numérique |  | N | Sum of interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | interruptibleAggregatedCos |
| 14 | ∑COSi A | Numérique |  | N | Sum of annual interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | annualInterruptibleAggregatedCos |
| 15 | ∑COSi T | Numérique |  | N | Sum of quarterly interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | quarterlyInterruptibleAggregatedCos |
| 16 | ∑COSi M | Numérique |  | N | Sum of monthly interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | monthlyInterruptibleAggregatedCos |
| 17 | ∑COSi Q | Numérique |  | N | Sum of daily interruptible subscribed operational capacities | 100,000000 (en GWh/J 25°C) | dailyInterruptibleAggregatedCos |
| 18 | CTNf | Numérique |  | N | Firm nominal technical capacity | 100,000000 (en GWh/J 25°C) | firmNominalTechnicalCapacity |
| 19 | Travaux Petits Impacts / Low Impact Maintenance | Booléen |  | O | Low Impact Maintenance | 100,000000 (en GWh/J 25°C) | lowImpactMaintenance |
| 20 | Date et Heure de Mise à jour / Update date and time | Date/Heure | aaaa-mm-jjThh:mm:ssZ | O |  | 100,000000 (en GWh/J 25°C) | updateDateTime |

File example :

# API interface

API signature (yaml format) are available at url below :

Production environment:

https://api.ingrid.natrangroupe.com/publication/operations/v3/api-docs.yaml

Staging environment :

https://api.ingrid-stg.natrangroupe.com/publication/operations/v3/api-docs.yaml

Please get from your commercial contract client and secret required for connection.

Document Technical guide connection API provides further details for using APIs.