Technical Guide

Metering Notice and Statement

February 25th 2026

| Référence |  | GuideTechnique-BM-EN |  |  |  | Classement |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accessibilité |  |  | Accès réservé |  | Restreint |  |  | Interne |  |  | Libre (à préciser) | X |
| Résumé |  |  |  |  |  |  |  |  |  |  |  |  |
| This document describes the exchange format for the Metering Statement and Metering Notice. |  |  |  |  |  |  |  |  |  |  |  |  |

# Versions list

| Version | Date | Auteur(s) | Description |
| --- | --- | --- | --- |
| V0.1 | 25/01/2022 | A Soudé | First Version |
| V0.2 | 10/10/2023 | A Soudé | URL API Information §6 |
| V0.3 | 20/10/2023 | A Soudé | AM file example |
| V0.4 | 25/02/2026 | K. GAID | Change of company name. Effective as of 01st July 2026. |

# File definition

Metering Statements or Metering Notices are files dedicated to publish the metering data to our clients.

- The Metering Notice (AM) contains the data for a single gasday : daily and hourly measures for the previous gasday

- The Within Day Metering Notice (AMJ) contains all the hourly measures for the current gasday

Metering Statements contains data for a whole month.

- Provisional Metering Statements (BMP) contains the daily and hourly measures for the current month (for the 1st of the month to D-1)

- Definitive Metering Statements (BMD) contains the daily and hourly measures for the last month

- Rectified Metering Statements (BMR) contains the daily and hourly measures for months older than the last one  (for M-2 to M-18)

These files contains the following data :

- The energy, volume and GCV for the gasday and each hour of the gasday

- The gas quality details

# Where to find the files

The files are provided by NaTran for the shippers (for each transmission contract) and for the industrial customers (for each connection contract). They are made available the following ways :

- Proactively :

  - A CSV file is available and can be downloaded on our ingrid website

  - The same CSV file is also available on our sFTP server (the Technical Guide for the sFTP connection is available at NaTran.com)

  - An EDIG@S METRED v5.1 file is also available

- Via API :  Dedicated APIs are available to request the metering data. Daily and hourly data are shown in separate API. The interface contract for the API is described in §6. You can contact your usual operational contact to gain access to our API.

# File name and format

The files are published in the CSV format, with :

- A semicolon as a list separator

- A comma as a decimal mark

The files will be named according to the following rule :

| N° | Designation | Type | Length | Format |
| --- | --- | --- | --- | --- |
| 1 | Document type | Alphanumeric | 2 or 3 | AMJ/ AM/ BMP/ BMD/ BMR |
| 2 | Contrat code | Alphanumeric | Up to 30 |  |
| 3 | Gasday (AM / AMJ) Gas month (BMP, BMP, BMR) | Date | 8 6 | YYYYMMDD YYYYMM |
| 4 | Date | Horodate | 17 | DDMMYYYYhhmmssSSS |
| 5 | Extension | Alphanumeric | 4 | .csv |
| 6 | Separators |  | 3 | « _ » |

Therefore, the file names are:

AMJ_CONTRACTCODE_YYYYMMDD_DDMMYYYYhhmmssSSS.csv

AM_CONTRACTCODE_YYYYMMDD_DDMMYYYYhhmmssSSS.csv

BMP_CONTRACTCODE_YYYYMM_DDMMYYYYhhmmssSSS.csv

BMD_CONTRACTCODE_YYYYMM_DDMMYYYYhhmmssSSS.csv

BMR_CONTRACTCODE_YYYYMM_DDMMYYYYhhmmssSSS.csv

# File description

The files contain 3 sections, each section separated by a line break :

- Header

- Daily data

- Hourly data

The following table describes the sections relevant to each specific file :

| File | Header | Daily data | Hourly data |
| --- | --- | --- | --- |
| AMJ | X |  | X |
| AM | X | X | X |
| BMP | X | X | X |
| BMD | X | X | X |
| BMR | X | X | X |

Section 1 - Header  :

The header lists information about the publication time  and the gasdays included in the file.

This section contains the following data :

- An ID :

  - For the AMJ :

    - Mesures intra-journalières / Within-Day Metering Notice

      - Example : AMJ-XXXXX

  - For the AM :

    - Avis de Mesure / Metering Notice :

      - Example : AM-XXXXX

  - For the BMP :

    - Bordereaux de mesure provisoire / Provisionnal Metering Statement

      - Example : BMP-XXXXX

  - For the BMD :

    - Bordereaux de mesure définitif / Definitive Metering Statement

      - Example : BMD-XXXXX

  - For the BMR :

    - Bordereaux de mesure redressé / Rectified Metering Statement

      - Example : BMR-XXXXX

- Réseau / Network :

  - Constant value =  NaTran

- Période / Period :

  - Example : 01/12/2022 06:00 –  02/12/2022 06 :00

  - The period represents the first and last gasdays included in the file

- ID contrat/ ID contract:

  - Example : GFXXXX01

- ID expéditeur/ ID shipper :

  - In case of a transmission contract :

    - The shipper ID is made with the contract ID without the last 2 digits

    - Example : GFXXXX

  - In case of a connection contract or a holding  :

    - The shipper ID is made with « SOC » followed by 4 or 5 digits.

    - Example : SOCXXXXX

- Nom de l’expéditeur / Name of the shipper :

  - Example : XXXX

  - Company name

- Date de mise à jour / Last update

  - Example : 02/12/2021 01:16:25

  - Publication time

Sections 2 and 3 – Daily and Hourly data:

Daily table:

This table is not included in AMJ files.

The table (with semicolon list separators) includes :

- A header line with the name of each columns

- The daily data with a different line per (PCE / Gasday/ Type of metering)

| N° Column | Column label | Type | Format | Mandatory | Description | Example of possibles values |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Code PCE / Metering point code | Text |  | Y | PCE code | ACI50170 |
| 2 | Libellé PCE / Metering point label | Text |  | Y | PCE label | BANEUIL POLYREY |
| 3 | Journée gazière / Gasday | Date | DD/MM/YYYY | Y | Gasday | 25/01/2023 |
| 4 | Type de mesure / Metering type | Text |  | N | Type of metering | VOLUME ENERGIE PCS DENSITE MASSE TX_CO2 TX_N2 TX_CH4 INDICE_WOB TX_C2H6 TX_C3H8 TX_C4H10_ISOB TX_C4H10_NBU TX_C5H12_NEOP TX_C5H12_ISOP TX_C5H12_NPE TX_C6P TX_HYD |
| 5 | Unité / Unit | Text |  | N | Unit | m3(n) kWh à 0°C kWh à 0°C/m3(n) Sans unité kg/m3(n) Pourcentage |
| 6 | Date et heure de mise à jour / Update date and time | Horodate | YYYY-MM-DDTHH:MM:SSZ | N | Update time | 2023-01-25T17:30:00Z |
| 7 | Valeur/ Value | Figure |  | N | Metering value | 11,435 |
| 8 | Qualité / Quality | Text |  | N | Metering quality | A : Acquis AF: Fiable AD: Douteux RI : Remodulé à partir d’un index RF : Remodulé suite à un forçage EM : Estimé manuellement EA : Estimé automatiquement AA : Acquis en alarme F3 : forçage type 3 Bp : forçage bipasse FJ : forçage journalier D : Douteux M : Manquant -: Non connu B : Backup |

Hourly table :

The table (with semicolon list separators) includes :

- A header line with the name of each columns

- The hourly data with a different line per (PCE / Gasday/ Type of metering/hour)

| N° Column | Column label | Type | Format | Mandatory | Description | Example of possible values |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Code PCE /Metering point code | Text |  | Y | PCE code | ACI50170 |
| 2 | Libellé PCE / Metering point label | Text |  | Y | PCE label | BANEUIL POLYREY |
| 3 | Journée gazière / Gasday | Date | DD/MM/YYYY | Y | Gasday | 25/01/2023 |
| 4 | Type de mesure / Metering type | Text |  | N | Type of Metering | VOLUME ENERGIE PCS DENSITE MASSE TX_CO2 TX_N2 TX_CH4 INDICE_WOB TX_C2H6 TX_C3H8 TX_C4H10_ISOB TX_C4H10_NBU TX_C5H12_NEOP TX_C5H12_ISOP TX_C5H12_NPE TX_C6P TX_HYD |
| 5 | Unité / Unit | Text |  | N | Unit | m3(n) kWh à 0°C kWh à 0°C/m3(n) Sans unité kg/m3(n) Pourcentage |
| 6 | Date et heure de mise à jour / Update date and time | Horodate | YYYY-MM-DDTHH:MM:SSZ | N | Update time | 2023-01-25T17:30:00Z |
| 7 | Valeur / Value | Figure |  | N | Value | 100 000 |
| 8 | Qualité / Quality | Text |  | N | Quallity | A : Acquis AF: Fiable AD: Douteux RI : Remodulé à partir d’un index RF : Remodulé suite à un forçage EM : Estimé manuellement EA : Estimé automatiquement AA : Acquis en alarme F3 : forçage type 3 Bp : forçage bipasse FJ : forçage journalier D : Douteux M : Manquant -: Non connu B : Backup |
| 9 | Heure / Hour | Hour | HH :MM | Y | Hour | 06:00 |

File examples :

AMJ  Example:

AM Example:

BMP/BMD/BMR  Example:

# API interface contract

The format of the API (yaml) is available in the below url:

For the production environment:

https://api.ingrid.natrangroupe.com/publication/realisations/v3/api-docs.yaml

For the staging environment

https://api.ingrid-stg.natrangroupe.com/publication/realisations/v3/api-docs.yaml

The access to this API requires an authentication (client and secret). They need to be asked to your operational contact.

The API connection technical guide explain on website natrangroupe.com the way to use these API.